# Spring-Compensated Floating O-Ring Retainer for Vibration-Resilient Carousel Seal

Date: 2026-07-20
Reference: Self-compensating rotary dynamic seal device, method and device assembly method
TRIZ: Parameter Compensation (Principle #15)

Cross-domain application: Each carousel cup O-ring is mounted in a floating annular retainer backed by a wave spring that applies continuous axial preload (5–20 N) toward the cup sealing surface.

# Original Problem

Seal force loss (compression set): Under 50–400 Hz vibration, the O-ring undergoes viscoelastic compression set, reducing radial contact pressure and compromising sub-micron aerosol seal integrity.

# Core Modification

Floating Retainer + Wave Spring: As the O-ring relaxes under vibration, the spring automatically advances the retainer to restore radial contact pressure.

# Axial Motion (Auto-Compensation)

Spring advances retainer to restore seal force as O-ring relaxes under vibration.

# Local Magnified View — Sealing Interface

| Component | Function |
| --- | --- |
| Cup Sealing Surface | Target sealing interface |
| O-Ring | Primary seal element |
| Floating Retainer | Holds O-ring, moves axially under spring force |
| Wave Spring | 5–20 N axial preload, auto-compensates for compression set |
| Retainer Ring (snap or threaded) | Secures retainer assembly, enables <5 min field swap |

# Validation / Parameter Window (To Be Validated)

| Parameter | Target / Status |
| --- | --- |
| Vibration Range | 50–400 Hz |
| Spring Preload | 5–20 N (axial) |
| O-Ring Material Durometer | To be validated |
| Retainer Interface (snap or thread) | To be validated |

# System Impact

Added mass: ~50 g per cup (~600 g total for 12 cups)

No power or electronics required — fully passive

Field swap: <5 min per cup

Integrates into existing cup-lid interface

Maintains sub-micron aerosol seal integrity under continuous vibration

Note: All parameters to be validated.