# Dual-Layer Piston-Ring and Elastomer Seal for Vibration and Shock-Resilient Carousel Isolation

Date: 2026-07-20
Reference: A novel double-layer dustproof and leak-proof sealed turntable bearing
TRIZ: Segmentation (Principle #1)
Analysis Method: Substance-field analysis

Cross-domain application: Each carousel cup rim is machined with a stepped groove housing a dual-layer seal: an inner PTFE piston ring provides low-friction primary sealing under 50–400 Hz steady-state vibration, while an outer fluoroelastomer O-ring in a W-shaped expansion groove absorbs 2–5 g percussion shock impulses and compensates for radial offset.

# Original Problem

Leakage path & bottleneck: Seal lift-off and radial offset under vibration and shock create aerosol leakage paths, compromising biohazard isolation.

# Core Modification

Dual-layer seal cartridge in stepped groove: No disassembly required for field replacement.

# Seal Layer Detail

| Layer | Material / Geometry | Function |
| --- | --- | --- |
| Inner (Primary) | PTFE piston ring | Low-friction sealing under 50–400 Hz vibration |
| Outer (Secondary) | FKM O-ring in W-groove | Absorbs 2–5 g shock impulses, compensates radial offset |

# How It Solves & System Impact

Dual-layer redundancy maintains sub-micron aerosol isolation even if one layer degrades

Snap-fit cartridge enables <5 min field replacement (no disassembly)

Added mass: ~80 g per cup (960 g total for 12 cups)

No power, no active control — fully passive

Fits within existing cup geometry

# Validation Area (To Be Validated)

| Validation Item | Concern |
| --- | --- |
| PTFE piston-ring wear rate | Under continuous vibration |
| Wear debris migration | Into MinION sample path |
| Compression set & seal lift-off | Over operational life |
| Long-term chemical compatibility | With sample materials and cleaning agents |

Note: Parameter window and test boundary to be established.