# Inline Expansion Chamber for Pulse Damping and Shear Reduction

Date: 2026-07-20
Reference: Noise reduction and desorption system and vehicle
TRIZ: Space separation (based on Principle #1 Segmentation and Principle #35 Parameter Change)
Analysis Method: Physical contradictions

Cross-domain application: Install a passive expansion chamber (40–60 mm diameter, 100–150 mm length) between the 12 mm transport tube exit and the cyclone separator inlet.

# Original Problem

High velocity (10–20 m/s): Excessive kinetic energy and shear stress damage fragile biological samples while failing to adequately break mineral aggregates.

# Core Modification

Passive expansion chamber (Ø40–60 mm, L 100–150 mm): Sudden cross-section increase decelerates flow from 10–20 m/s to 2–5 m/s, dissipating kinetic energy and smoothing pressure pulses.

# Action Path (Mechanism)

Flow enters expansion chamber — sudden area increase

Deceleration & energy dissipation via turbulent mixing

Mineral fragments & biologicals continue at reduced velocity

Enter cyclone at reduced peak velocity (2–5 m/s)

# Bottleneck Solved

Velocity contradiction (inlet to cyclone): High shear & pressure pulses vs. gentle tissue handling. Resolved passively by spatial separation along the flow path.

# System Impact

Passive structure: <1 kg (sheet metal or PEEK)

Fits within ≤Ø400 × 350 mm envelope

Integrates into PlanetVac interface

No active control or added power required

Uses existing N₂/CO₂ kinetic energy

# Validation Area (To Be Confirmed)

| Parameter | Validation Need |
| --- | --- |
| Chamber volume & baffle geometry | Optimize for flow deceleration and energy dissipation |
| CFD verification | Velocity field confirmation across operating range |
| Benchtop test | Biological-tissue protection & mineral-fragment passage without secondary settling or clogging |

Note: Parameters above are windows to be validated.