# Vision-Based Real-Time Pose Calibration for Multi-Interface Robotic Sample Delivery

Date: 2026-07-20
Reference: A calibration system, method, device and medium for a robotic arm end effector
TRIZ: Principle #26 (Copying), Principle #24 (Intermediary)
Analysis Method: Functional modeling

Cross-domain application: Mount passive optical markers on the delivery arm end-effector and on fixture plates at each of nine instrument interfaces; NVIDIA Jetson Isaac ROS2 stereo camera continuously tracks marker poses and computes transformation matrices.

# Original Problem

Pose errors from backlash, drift, vibration: Encoder quantization, backlash, thermal drift, and vibration deflection cause accumulated positioning error without extra actuators or changing end-effector geometry.

# Solution Change

Add passive optical markers + vision tracking: Before each sample delivery, the control loop updates arm trajectory with corrected coordinates.

# Action Path

Stereo camera tracks marker poses (Jetson Isaac ROS2)

Compute transform T_Base^Interface

Update trajectory with corrected target coordinates

Arm controller executes corrected motion

Achieve ±0.1 mm MinION precision

# System Architecture

| Component | Function | Details |
| --- | --- | --- |
| Stereo Camera (Jetson Isaac ROS2) | Vision sensing | Tracks passive markers in real time |
| Passive markers (end-effector) | Reference frame | Modified point on end-effector |
| Passive markers (9 fixtures) | Target frames | MinION, LIBS, Raman, etc. |
| Jetson (Isaac ROS2) | Sensor fusion & compute | Marker tracking + transform computation |
| Control Loop (ROS2) | Trajectory correction | Update trajectory with T_Base^Target |
| Arm Controller | Motion execution | Executes corrected motion to target interface |

# Key Benefits

<5 min instrument swap

No external recalibration required

Uses existing Jetson + ROS2 stack

Marker mass <50 g per fixture

Calibration persists across field swaps when fixtures reinstalled with markers

# Validation Area (Parameter Window)

| Parameter | Validation Need |
| --- | --- |
| Lighting & Visibility | Marker detection under varying illumination |
| Marker Placement & Attachment | Repeatable mounting for consistent pose accuracy |
| Vibration & Dynamics | Tracking stability under 50–400 Hz vibration |
| Accuracy Window | ±0.1 mm for MinION precision |

Note: Marker placement, environment, and accuracy parameter window to be validated.