# Passive Compliant End-Effector Mount with Contact-Force Self-Centering for Multi-Interface Alignment

Date: 2026-07-20
Reference: A purge mechanism for wafer loader
TRIZ: Introduce flexible mechanical field (spring-damper compliance) to absorb positioning error and enable self-centering
Analysis Method: Substance-field analysis

Cross-domain application: Replace the rigid arm-to-end-effector coupling with a floating compliant mount that passively self-centers via contact force from interface geometry features.

# Original Problem

Rigid coupling passes arm positioning error to interface: ±0.5–0.8 mm accumulated error causes misalignment, spillage/damage risk, and vibration transmission.

# Core Modification

Floating compliant mount (±1 mm / ±3° travel): Linear springs (~10 N/mm) and torsion springs (±3° tilt) allow passive self-centering.

# Mechanism Detail

| Component | Specification | Function |
| --- | --- | --- |
| Linear springs | ±1 mm axial travel, ~10 N/mm stiffness | Axial compliance and preload |
| Torsion springs | ±3° angular tilt | Angular compliance for tilted approach |
| Fail-safe lock | Pneumatic or magnetic | Rigidifies mount after alignment for transfer |
| Metal sleeve | Hard stop | Limits max displacement under 15 Nm torque reaction |

# Action Path (Normal Sequence)

Approach — Compliant floating

Passive Self-Centering — Aligned to ±0.1 mm via interface geometry (USB-C slot chamfer, Raman ferrule taper, etc.)

Lock & Transfer — Fail-safe lock engages, mount rigidifies

Unlock & Retract — Lock releases before retraction

# System Impact

Added mass: ~200 g

No active control or power draw (lock uses existing 100 PSI pneumatic supply)

Compliant mount isolates 50–400 Hz drilling vibration from end-effector

Self-centering to ±0.1 mm from ±0.5–0.8 mm arm error

# Validation Boundary (Key Unknowns)

| Validation Item | Description |
| --- | --- |
| All nine interface geometries | Must provide sufficient centering force |
| Locking mechanism | Must not introduce new failure modes under biohazard sample handling |
| Long-term reliability | Under repeated lock/unlock cycles |

Note: Largest validation gap is ensuring all nine interface geometries provide sufficient centering force.