# Modular Kinematic Adapter Plates for Universal End-Effector Multi-Interface Sample Delivery

Date: 2026-07-20
Reference: Repeated positioning precision testing fixture of high-precision connecting device

TRIZ: System level separation (based on Principle #1 Segmentation and Principle #6 Universality)
Analysis Method: Physical contradictions

Cross-domain application: Design a universal end-effector with kinematic coupling interface (three precision balls) and passive adapter plates at each of nine instrument ports.

# What the Solution Changes

Standardized kinematic interface + passive adapter plates: Coarse position → Self-center → Interface-specific guidance.

# System Architecture

| Component | Specification | Function |
| --- | --- | --- |
| Universal End-Effector | Three precision balls | Coarse position by arm (±0.5 mm) |
| Adapter Plate (arm-facing) | Three V-grooves (120°) | Kinematic coupling self-center to ±0.05 mm |
| Adapter Plate (instrument-facing) | Interface-specific geometry | USB-C, Raman ferrule, FTIR funnel, LIBS cone, etc. |
| Instrument with 9 Ports | Pre-installed adapter plates | Each port has dedicated adapter |

# Example Adapter Plates (9 Total)

USB-C Data Port

Raman Ferrule Socket

FTIR Sample Cup Funnel

LIBS Laser Alignment Cone

... 5 additional instrument-specific plates

# System Impact

Passive plates: ~50 g each (~450 g total at instrument side, not on arm mass budget)

Purge gas cleans plate surfaces between samples

Field swap: <5 min (arm module + 9 plates if pre-installed)

No active control or power required

# Validation Area (Largest Gaps)

| Validation Item | Description |
| --- | --- |
| Optical / fluidic clearance | For Raman & LIBS adapter plates |
| Swap time budget | <5 min including 9 plates (if pre-installed) |

Note: Largest validation gap is optical/fluidic clearance for Raman and LIBS adapter plates and whether swap time budget accommodates plate installation.