# Dual-Motor Segmented Torque Architecture with Sample-Type Adaptive Distribution for 0.1–15 Nm Range

Date: 2026-07-20
Reference: A torque distribution method for a novel dual-motor structure in an energy electric vehicle

TRIZ: Segmentation (Principle #1)
Analysis Method: Technical contradictions

Cross-domain application: Replace the single BLDC drill with two independent motors driving a common drill chuck via torque-summing differential or sequential one-way clutches.

# Original Bottleneck

Single BLDC drill cannot optimize for both tissue and mineral: Low-inertia fast response vs. high-torque percussion are mutually exclusive in a single motor.

# Core Modification

Dual-motor segmented torque architecture:

# Motor Configuration

| Motor | Type | Torque / Response | Function |
| --- | --- | --- | --- |
| Biological Motor | Coreless + 10:1 planetary | 0.1–0.5 Nm, <10 ms response | Low-inertia tissue sampling |
| Mineral Motor | BLDC + 50:1 planetary | 5–15 Nm | High-torque percussion drilling |
| Elastomeric coupling | Shock isolation | Isolates 2–5g shocks | Prevents shock transmission to seal/MiniON |

# Control & Decision Path

Real-time sensor fusion: Axial force + Motor current + Acoustic signature

Jetson (ROS2) sample classification

Torque distribution firmware (1 kHz) — <20 ms mode switching

Selects the right motor, idles the other (power save)

# System Impact

Total added mass: ~600 g (two small planetaries and couplings)

Power stays within 200 W by duty-cycle interleaving (only one motor active per sample)

Target object: Sample (biological tissue or mineral core)

Performance goal: <20 ms mode switching

# Validation Area (To Be Validated)

| Validation Item | Description |
| --- | --- |
| Packaging feasibility | Dual planetary stages within Ø400×350 mm drill mount |
| Shock isolation performance | Elastomeric coupling under 50–400 Hz vibration, 2–5g |
| Firmware & classification | Integration with Isaac ROS2 sample classification |

Note: Requires iterative prototyping & tests. All are standard mechatronic engineering tasks.