# Hollow Conical Rubber Isolator for Percussion Shock Attenuation Between Drill and Frame

Date: 2026-07-20
Reference: Vehicle-mounted vibration isolation optical platform
TRIZ: Mechanical field evolution to damping field
Analysis Method: Substance-field analysis

Cross-domain application: Replace the rigid bolted interface between the BLDC drill motor housing and the DrACO frame with four hollow conical rubber isolators, one at each mounting corner.

# Original Bottleneck

Rigid bolted interface transmits percussion shock (2–5g) and high-frequency vibration: Shock and vibration propagate to carousel seal and MiniON zone.

# Core Modification

Conical rubber isolation mounts (×4): Progressive stiffness — compliant enough to absorb shocks, stiff enough to maintain drill tip positioning accuracy within 0.5 mm.

# Isolator Detail

| Component | Function | Specification |
| --- | --- | --- |
| Conical rubber (isolator) | Shock absorption & vibration damping | Progressive stiffness, hysteresis damping |
| Metal sleeve | Hard stop & creep resistance | Limits max displacement under 15 Nm torque reaction |
| Material | Nitrile rubber or fluorosilicone | Resistant to oil, mineral dust, 40°C ambient |

# System Impact

Converts shock kinetic energy to heat via elastomer hysteresis

Natural frequency tuned below 30 Hz (avoids 50–400 Hz vibration band)

Maintains drill tip positioning accuracy within 0.5 mm for Raman/LIBS alignment

Protects carousel seal and MiniON zone from percussion shock transmission

# Validation Area (To Be Validated)

| Validation Item | Description |
| --- | --- |
| Stiffness tuning window vs. positioning accuracy | ≤0.5 mm target |
| Natural frequency target | <30 Hz, avoiding 50–400 Hz band |
| Material durability | Debris (oil, mineral dust) & 40°C ambient |
| Continuous high-torque endurance & long-term creep | Under repeated percussion loads |
| Shock attenuation vs. heat build-up | In elastomer under continuous operation |

Note: All are standard elastomer engineering tasks but require iterative prototyping and accelerated life testing.