EMBEDDED HEAT PIPE THERMAL HIGHWAY

Passive Shell-Surface Heat Rejection for Sealed Pod

Saturday, July 11, 2026

| Document ID | THERM-HP-001 |
| --- | --- |
| Revision | Rev. A — Draft for Review |
| Date | 2026-07-11 |
| Classification | Engineering Technical Document |

# 1. EXECUTIVE SUMMARY

This document presents an embedded heat pipe thermal highway solution for passive heat rejection from a sealed carbon fiber pod. Two to three copper heat pipes (Ø6–8 mm, 50–80 mm length, sintered wick, water working fluid, ~50–80 g each) are routed from internal hot zones (TEC cold-side, Jetson heatsink) to the carbon fiber shell top/side surfaces via sealed IP54-rated grommet fittings. Phase-change vapor transport achieves effective thermal conductivity of ~10,000–50,000 W/m·K with thermal resistance ~0.1–0.2 K/W, moving 20–30 W per pipe (30–50 W total) from the sealed interior to external shell surfaces. The shell acts as a passive radiator (0.44 m² area, natural convection ~5–10 W/m²·K, emissivity ~0.8), reducing interior temperature rise by 10–15 °C compared to the sealed baseline. This higher-order thermal field replaces low-order conduction, keeping TEC within rated 15–20 W capacity and QCL/MinION within operating limits without active cooling hardware, power increase, or seal breach. Total added mass is <0.2 kg, with <5 min field serviceability.

# 2. PROBLEM STATEMENT

The sealed carbon fiber pod suffers from heat accumulation due to inadequate thermal transport from internal hot zones to ambient:

Heat accumulates inside sealed pod — 75–80 W total thermal load from QCL, MinION, TEC, and Jetson modules with no efficient escape path.

Low-order conduction bottleneck — heat must conduct through carbon fiber (k ~1–5 W/m·K) and air gaps, creating high thermal resistance.

TEC saturation — thermoelectric coolers rated for 15–20 W cannot handle the full thermal load when heat cannot escape the enclosure.

Temperature rise excessive — interior temperatures exceed QCL (<45 °C) and MinION (<40 °C) operating limits.

Sealed enclosure constraint — IP54-rated pod cannot be opened for ventilation or active cooling during operation.

Active cooling prohibited — fans, pumps, or liquid cooling add mass, power, complexity, and failure modes incompatible with field deployment.

Shell underutilized — the large carbon fiber shell surface (0.44 m²) has high emissivity and convective potential but is thermally isolated from internal heat sources.

Goal: Create a high-efficiency thermal highway from internal hot zones to the external shell surface, reducing interior temperature rise by 10–15 °C without active cooling, power increase, or seal breach.

# 3. SOLUTION ARCHITECTURE

## 3.1 Heat Pipe Thermal Highway Layout

The heat pipe array creates a near-isothermal thermal highway from internal heat sources to the external shell radiator:

| Heat Pipe | Hot End (Evaporator) | Cold End (Condenser) | Heat Transport | Route |
| --- | --- | --- | --- | --- |
| HP-1 | TEC cold-side plate | Shell top surface | ~20–30 W | Vertical rise from TEC to top shell |
| HP-2 | Jetson heatsink base | Shell side surface | ~20–30 W | Horizontal run from Jetson to side shell |
| HP-3 (opt.) | QCL mounting plate | Shell top/side surface | ~10–20 W | Short run from QCL to nearest shell surface |

## 3.2 Phase-Change Transport Action Path

Each heat pipe operates through a self-sustaining phase-change cycle that achieves orders-of-magnitude higher thermal conductivity than solid conduction:

Evaporation (Hot End): Heat from the TEC cold-side or Jetson heatsink vaporizes the working fluid (water) in the evaporator section. The sintered copper wick provides nucleation sites and capillary structure. Evaporation occurs at ~40–50 °C (saturation temperature at internal pressure). Latent heat of vaporization (~2260 kJ/kg) absorbs large amounts of heat with minimal temperature rise.

Vapor Flow (Low Resistance): Vapor flows through the hollow core of the heat pipe from evaporator to condenser. The pressure differential is minimal (<100 Pa) because vapor density is low and flow resistance is negligible. This near-zero thermal resistance is the key advantage over conduction.

Condensation (Cold End): At the condenser section bonded to the carbon fiber shell, vapor condenses on the inner wall, releasing latent heat. The condensate wets the sintered wick and is drawn back to the evaporator by capillary action. Condensation heat transfer coefficient is very high (~5000–10,000 W/m²·K), enabling efficient heat rejection to the shell.

Capillary Return (Wick): The sintered copper wick (pore size 50–100 µm, porosity 50–60%) provides capillary pressure to return condensate against gravity. For vertical orientation (evaporator below condenser), gravity assists return. For horizontal or adverse tilt, capillary pressure alone must overcome viscous losses. The wick is designed for maximum capillary pressure while minimizing flow resistance.

Heat Rejection to Ambient: The shell condenser region acts as a passive radiator. Heat is rejected via natural convection (h ~5–10 W/m²·K) and radiation (ε ~0.8, σ = 5.67×10⁻⁸ W/m²·K⁴) to ambient. The large shell area (0.44 m²) provides adequate heat rejection capacity for 30–50 W.

## 3.3 Higher-Order Thermal Field

The heat pipe introduces a higher-order thermal field that fundamentally transforms the pod's thermal behavior:

Low-Order Field (Baseline): Conduction through carbon fiber (k ~1–5 W/m·K) and air gaps. Thermal resistance R = L/(kA) is high due to low conductivity and long path lengths. For L = 100 mm, k = 2 W/m·K, A = 0.01 m²: R = 5 K/W. This creates large temperature gradients and slow response.

Higher-Order Field (Heat Pipe): Phase-change transport with effective thermal conductivity k_eff ~10,000–50,000 W/m·K. For the same geometry: R = 0.1–0.2 K/W. This is a 25–50× reduction in thermal resistance, enabling near-isothermal transport over long distances.

Su-Field Analysis: Substance-field model: S1 (heat source) — F1 (low-order conduction field) — S2 (shell) is ineffective (incomplete field). Introducing F2 (phase-change vapor field) creates: S1 — F2 (phase change) — S2. The higher-order field completes the functional interaction, transforming the shell from a passive barrier into an active radiator.

Synergistic Effect: The heat pipe does not replace the shell — it activates it. The shell's 0.44 m² surface area, high emissivity, and natural convection potential become functional heat rejection capacity when thermally coupled to internal sources via the heat pipe highway.

# 4. DESIGN DETAILS

## 4.1 Heat Pipe Specifications

Each heat pipe is a custom or off-the-shelf copper-water heat pipe optimized for the pod thermal environment:

| Parameter | Specification |
| --- | --- |
| Material | Oxygen-free copper (C10200) tube, wall thickness 0.3–0.5 mm |
| Outer Diameter | Ø6–8 mm (to be validated: 6 mm for tight spaces, 8 mm for higher capacity) |
| Length | 50–80 mm (evaporator + adiabatic + condenser sections) |
| Wick Structure | Sintered copper powder, pore size 50–100 µm, porosity 50–60%, thickness 0.5–1.0 mm |
| Working Fluid | Deionized water (high latent heat, compatible with copper, non-toxic) |
| Fluid Charge | ~0.5–1.0 g (optimized for pipe volume and operating temperature) |
| Operating Temperature | 30–70 °C (water heat pipes effective from ~20 °C to ~150 °C) |
| Max Heat Transport | 20–30 W per pipe (dependent on diameter, length, and orientation) |
| Thermal Resistance | ~0.1–0.2 K/W (evaporator-to-condenser, including wall and wick) |
| Effective Thermal Conductivity | ~10,000–50,000 W/m·K (equivalent solid conductivity) |
| Mass per Pipe | ~50–80 g (copper tube + wick + water) |
| Orientation | Vertical preferred (gravity-assisted); horizontal acceptable with optimized wick |

## 4.2 IP54 Grommet Fitting Design

The heat pipe penetration through the shell must maintain IP54 ingress protection while minimizing thermal resistance:

Grommet Material: Silicone rubber or EPDM, Shore A 40–60, temperature rated to 100 °C

Seal Design: Double-lip radial seal with internal compression spring. The heat pipe outer surface is smooth (Ra < 0.8 µm) to ensure seal contact.

Thermal Interface: Thermal grease (k > 3 W/m·K) between heat pipe and grommet inner bore to minimize contact resistance. Alternatively, conductive silicone grommet (k > 1 W/m·K) combines sealing and heat transfer.

IP54 Validation: Dust: no ingress of particles >10 µm. Water: no ingress from splashing (IPX4). Grommet design verified by pressure decay test (<1 Pa/min leakage at 500 Pa differential) and IP54 compliance test per IEC 60529.

Mechanical Retention: Grommet retained in shell by external flange and internal snap ring. Heat pipe slides through grommet with light interference fit.

Vibration Tolerance: Grommet accommodates ±2 mm relative motion between heat pipe and shell under 50–400 Hz vibration without seal degradation.

## 4.3 Shell Radiator Performance

The carbon fiber shell acts as a passive radiator when thermally activated by the heat pipe condensers:

Radiator Area: 0.44 m² (total external surface area of pod shell)

Surface Emissivity: ε ~0.8 (carbon fiber composite with matte finish; can be enhanced with black anodized coating or high-emissivity paint)

Natural Convection: h_conv ~5–10 W/m²·K (vertical/horizontal surfaces in still air; wind increases to 15–25 W/m²·K at 5 m/s)

Radiation Heat Rejection: Q_rad = εσA(T_s⁴ – T_a⁴). For T_s = 55 °C (328 K), T_a = 30 °C (303 K): Q_rad = 0.8 × 5.67×10⁻⁸ × 0.44 × (328⁴ – 303⁴) ≈ 50 W.

Convection Heat Rejection: Q_conv = hA(T_s – T_a). For h = 7.5 W/m²·K, ΔT = 25 °C: Q_conv = 7.5 × 0.44 × 25 ≈ 83 W.

Total Rejection Capacity: Q_total ≈ 50 + 83 = 133 W theoretical maximum. Practical limit (with condenser area constraints, non-uniform temperature): 30–50 W.

Temperature Reduction: With 30–50 W rejected via shell radiator, interior heat load on TEC is reduced from 75–80 W to 45–55 W. TEC operates within 15–20 W rated capacity, and interior temperature rise is reduced by 10–15 °C.

## 4.4 Structural & Mass Budget

The heat pipe system adds minimal mass and integrates with existing pod structure:

| Parameter | Specification |
| --- | --- |
| Heat Pipes (×3) | ~150–240 g total (50–80 g each) |
| Grommet Fittings (×3) | ~30–45 g total (silicone/EPDM + metal retainers) |
| Thermal Interface Material | ~10–20 g (thermal grease or pads at evaporator and condenser) |
| Mounting Hardware | ~20–30 g (clips, brackets, screws) |
| Total Added Mass | <0.2 kg (well within ≤12 kg pod mass budget) |
| Power Consumption | Zero — passive phase-change transport requires no electrical power |
| Volume Occupied | ~3× Ø8 mm × 80 mm = ~12 cm³ internal volume (negligible vs. pod volume) |

# 5. INTEGRATION & FIELD SERVICEABILITY

## 5.1 Structural Integration

The heat pipe system integrates with existing pod components without major redesign:

Evaporator Mounting: Heat pipe evaporator sections are clamped or bonded to TEC cold-side plate and Jetson heatsink base using thermal interface material (TIM). Existing mounting holes and brackets are leveraged where possible.

Routing: Heat pipes follow existing cable raceways or structural rib channels to minimize interference with other components. Bend radius ≥3× pipe diameter (18–24 mm) to prevent wick damage.

Condenser Mounting: Heat pipe condenser sections are bonded to the inner surface of the carbon fiber shell using thermally conductive epoxy (k > 2 W/m·K). The shell surface is prepared by light abrasion and cleaning.

Grommet Installation: Shell penetration holes are drilled and reamed to grommet specification. Grommets are pressed into place with retaining flanges. Seal integrity verified by pressure decay test.

## 5.2 Field Service Procedure (<5 min target)

The modular heat pipe design enables rapid field replacement:

Step 1: Access Panel Removal
Open the pod quick-access panel. Heat pipe evaporator clamps and grommet retainers are accessible.

Step 2: Release Evaporator Clamp
Loosen the evaporator clamp screw (thumb screw or quarter-turn fastener). Slide heat pipe out of clamp.

Step 3: Release Grommet Retainer
Remove internal snap ring or loosen external flange screws. Slide heat pipe out of grommet.

Step 4: Install Replacement Heat Pipe
Slide new heat pipe through grommet (pre-lubricated with thermal grease). Secure retainer and evaporator clamp.

Step 5: Verify Seal
Perform visual inspection of grommet seating. Optional: pressure decay test if seal integrity is suspect. Total time: <5 min.

# 6. PERFORMANCE SPECIFICATIONS

| Parameter | Target | Verification Method |
| --- | --- | --- |
| Heat Transport per Pipe | 20–30 W | Calorimetry test with known heat load at evaporator |
| Total Heat Rejection | 30–50 W (2–3 pipes) | Sum of individual pipe calorimetry + shell surface thermography |
| Thermal Resistance | ~0.1–0.2 K/W per pipe | ΔT measurement between evaporator and condenser at rated load |
| Effective Thermal Conductivity | ~10,000–50,000 W/m·K | Calculated from R, L, A measurements |
| Temperature Reduction | 10–15 °C vs. sealed baseline | Comparative thermal test with thermocouple array |
| IP54 Sealing Reliability | Pass IEC 60529 | Dust chamber + water spray test after grommet installation |
| Radiator Performance | 0.44 m², ε~0.8, h~5–10 W/m²·K | Infrared thermography + heat flux sensors on shell surface |
| Added Mass | <0.2 kg | Precision balance |
| Power Consumption | Zero (passive) | Power meter verification |
| Field Service Time | <5 min (per pipe replacement) | Stopwatch, trained operator, 10 trials |

# 7. VALIDATION & TEST PLAN

## 7.1 Heat Pipe Performance Validation

Individual heat pipe performance shall be validated before system integration:

Thermal Resistance Test: Mount heat pipe in horizontal orientation with evaporator heated by resistive heater and condenser cooled by water block or heat sink. Apply 10 W, 20 W, 30 W loads. Measure ΔT between evaporator and condenser. Calculate R = ΔT/Q. Acceptance: R < 0.2 K/W at 30 W.

Max Heat Transport Test: Gradually increase heat load until evaporator dry-out (temperature spike). Record max sustainable load. Acceptance: >30 W per pipe.

Orientation Sensitivity: Test at 0° (horizontal), +45° (evaporator below), -45° (evaporator above), and +90° (vertical). Measure thermal resistance at each orientation. Acceptance: R < 0.3 K/W at all orientations (worst-case adverse tilt).

Thermal Cycling: Subject heat pipe to 1000 cycles between 30 °C and 70 °C with 20 W load. Re-measure thermal resistance. Acceptance: <10% degradation.

## 7.2 System-Level Thermal Validation

Integrated pod-level testing validates the complete thermal highway under realistic conditions:

Baseline Sealed Pod Test: Instrument baseline sealed pod with thermocouples at QCL, MinION, TEC cold-side, Jetson heatsink, and shell surfaces. Apply 75 W resistive heat load. Record steady-state temperatures. Establish baseline ΔT.

Heat Pipe-Enabled Pod Test: Repeat with heat pipes installed. Measure same temperatures. Calculate temperature reduction at each location. Acceptance: interior temperature rise reduced by 10–15 °C; shell surface temperature increased (confirming heat rejection).

Transient Response: Apply step heat load (0→75 W) and measure time to steady state. Compare baseline vs. heat pipe response times. Acceptance: heat pipe configuration reaches steady state faster due to lower thermal resistance.

Ambient Temperature Sweep: Test at 20 °C, 30 °C, 40 °C, 50 °C ambient. Verify heat pipe operation across range. At high ambient, reduced ΔT may decrease natural convection; verify TEC can handle residual load.

## 7.3 IP54 & Environmental Validation

Environmental robustness shall be validated per operational requirements:

IP54 Grommet Test: Install heat pipes with grommets in test shell. Perform IEC 60529 dust test (IP5X) and water spray test (IPX4). Inspect inner chamber for ingress. Acceptance: no dust or water ingress.

Vibration Test: Expose heat pipe assembly to random vibration (50–400 Hz, 5–20 g RMS, per axis, 8 h). Post-vibration: check grommet seal integrity, heat pipe thermal resistance, and mechanical retention. Acceptance: no seal degradation; R unchanged.

Thermal Shock: Subject pod to thermal shock (-20 °C ↔ +60 °C, 100 cycles). Verify grommet material does not crack or lose elasticity. Acceptance: no visible damage; pressure decay test passes.

## 7.4 Validation Parameter Window

The following parameters require experimental determination to finalize the design:

| Parameter | Current Estimate | Validation Method |
| --- | --- | --- |
| Pipe Diameter | Ø6–8 mm | Thermal resistance vs. diameter trade study |
| Pipe Length | 50–80 mm | Fit check in prototype pod; thermal resistance vs. length |
| Heat Transport per Pipe | 20–30 W | Calorimetry at multiple loads and orientations |
| Thermal Resistance | ~0.1–0.2 K/W | ΔT measurement at rated load |
| IP54 Sealing Reliability | Target: pass | IEC 60529 test after vibration and thermal shock |
| Radiator Performance | 0.44 m², ε~0.8, h~5–10 W/m²·K | IR thermography + ambient wind variation test |
| Temperature Reduction | 10–15 °C | Comparative thermal test: baseline vs. heat pipe pod |

# 8. RISK ASSESSMENT

| Risk | Likelihood | Impact | Mitigation |
| --- | --- | --- | --- |
| Heat pipe dry-out at high heat flux | Medium | High | Oversize pipe diameter; verify max heat transport; thermal fuse or temperature alarm |
| Grommet seal degradation over time | Medium | High | Accelerated aging test (85 °C/85% RH, 1000 h); redundant dual-lip seal; scheduled inspection |
| Non-condensable gas accumulation reducing performance | Low | High | Vacuum degassing during manufacture; hermetic pipe sealing; quality control |
| Wick degradation from vibration | Low | Medium | Vibration test; sintered wick bond strength verification; conservative safety margin on heat transport |
| Shell surface temperature too hot to touch | Medium | Low | Thermal warning label; shell temperature <60 °C under normal operation; insulation pad if needed |
| Condenser area insufficient for 30–50 W rejection | Medium | Medium | Shell surface area verification; enhanced emissivity coating; finned condenser option |
| Heat pipe corrosion in tropical humidity | Low | Medium | Copper-water compatibility (standard); internal passivation; external coating on exposed copper |

# 9. CROSS-DOMAIN INTEGRATION

This solution bridges thermal engineering, fluid mechanics, materials science, and systems design through a higher-order field transition:

Thermal Engineering: Heat pipe theory (two-phase heat transfer, capillary limit, boiling limit, sonic limit), thermal resistance network analysis, and radiator design (natural convection + radiation). The heat pipe transforms a distributed thermal resistance problem into a near-isothermal transport problem.

Fluid Mechanics: Vapor flow dynamics in narrow channels, condensate film flow, capillary pressure in porous media (Young-Laplace equation), and two-phase pressure drop correlations. The sintered wick design balances capillary pressure against viscous losses.

Materials Science: Copper tube and wick metallurgy (sintering parameters, porosity control), working fluid purity (deionized water, degassing), grommet elastomer selection (silicone vs. EPDM for temperature and chemical compatibility), and shell surface emissivity enhancement.

Systems Engineering: Mass, power, and volume budgets reconciled with thermal performance. Su-Field analysis confirms the transition from an incomplete low-order conduction field to a complete higher-order phase-change field. IP54 compliance maintained through seal engineering. Field maintainability (<5 min) integrated into modular design.

# 10. REFERENCES

[1] Vehicle Laser Radar Heat Dissipation Device — Patent / Process Reference Document.

[2] Faghri, A. 'Heat Pipe Science and Technology.' Taylor & Francis, 1995. (Comprehensive heat pipe theory and design)

[3] Peterson, G.P. 'An Introduction to Heat Pipes: Modeling, Testing, and Applications.' Wiley, 1994.

[4] Chi, S.W. 'Heat Pipe Theory and Practice.' Hemisphere Publishing, 1976. (Capillary limit and wick design)

[5] Incropera, F.P. & DeWitt, D.P. 'Fundamentals of Heat and Mass Transfer.' 7th Ed., Wiley, 2011. (Phase change and radiation heat transfer)

[6] IEC 60529:2013, 'Degrees of Protection Provided by Enclosures (IP Code).'

[7] MIL-STD-810G, 'Environmental Engineering Considerations and Laboratory Tests.' (Vibration and thermal shock methods)

[8] TRIZ Journal — Substance-Field Analysis and Higher-Order Field Transition. (Innovation methodology reference)

# 11. APPENDIX: SUBSTANCE-FIELD ANALYSIS & HIGHER-ORDER FIELD

## 11.1 Substance-Field (Su-Field) Analysis

The Su-Field model describes the interaction between substances (S) and fields (F) in the thermal management system:

Harmful Interaction (Original Problem): S1 = Internal heat sources (QCL, MinION, TEC, Jetson), S2 = Carbon fiber shell (potential radiator), F1 = Conduction field (low-order). The conduction field F1 is too weak to effectively transfer heat from S1 to S2. Heat accumulates in S1, causing temperature rise. This is an incomplete Su-Field because the field cannot complete the functional interaction.

Incomplete Su-Field (Baseline): S1 — F1 (conduction, k~1–5 W/m·K) — S2. The field is present but insufficient. The thermal resistance is too high, and the shell S2 remains thermally isolated from the heat sources S1.

Transition to Higher-Order Field (Solution): Introduce F2 = Phase-change vapor field (heat pipe) as a higher-order thermal field. The complete Su-Field becomes: S1 — F2 (phase change, k_eff~10,000–50,000 W/m·K) — S2. The higher-order field F2 replaces the insufficient field F1, enabling efficient heat transfer and completing the functional interaction.

Complete Su-Field Model: S1 (heat sources) — F2 (evaporation) → vapor flow → F2 (condensation) — S2 (shell radiator) — F3 (radiation + convection) → ambient. This is a complete, effective Su-Field model where all harmful interactions (heat accumulation) are neutralized by the higher-order field.

## 11.2 Higher-Order Field Characteristics

The phase-change heat pipe field exhibits characteristics that fundamentally distinguish it from conduction:

Field Order: Conduction is a first-order field (F1) governed by Fourier's law with linear temperature gradients. Phase-change transport is a second-order field (F2) governed by Clausius-Clapeyron relation with near-isothermal behavior and latent heat exchange.

Thermal Conductivity Enhancement: k_eff / k_solid = 10,000–50,000 / 2 = 5,000–25,000×. This is not merely an incremental improvement but a qualitative change in thermal transport mechanism — from phonon/electron diffusion to mass transfer of vapor.

Self-Regulation: The heat pipe is self-regulating: heat transport automatically matches heat input (up to capillary limit). No external control, sensors, or power are required. The field is 'intelligent' in that it adapts to thermal boundary conditions without active intervention.

Directionality: Unlike conduction, which spreads heat in all directions, the heat pipe provides directed transport from evaporator to condenser. This directionality enables precise thermal routing from hot zones to the shell radiator.

## 11.3 Parameter Changes from Baseline

The transition from low-order conduction to higher-order phase-change transport involves the following parameter changes:

| Parameter | Baseline (Conduction) | Modified (Heat Pipe) |
| --- | --- | --- |
| Thermal Transport Mechanism | Solid conduction (phonon/electron diffusion) | Phase-change vapor transport (latent heat + mass transfer) |
| Effective Thermal Conductivity | ~1–5 W/m·K (carbon fiber) | ~10,000–50,000 W/m·K |
| Thermal Resistance (100 mm path) | ~5 K/W | ~0.1–0.2 K/W |
| Heat Rejection from Shell | ~5–10 W (inefficient coupling) | ~30–50 W (direct thermal highway) |
| Interior Temperature Rise | 25–30 °C above ambient | 15–20 °C above ambient (10–15 °C reduction) |
| Active Cooling Required | Would be needed for adequate rejection | Not required (passive) |
| Added Mass | 0 kg (baseline) | <0.2 kg (3 heat pipes + grommets) |
| Power Consumption | 0 W (baseline) | 0 W (passive) |

# DOCUMENT CONTROL

| Prepared By | Thermal Engineering & Fluid Mechanics Team |
| --- | --- |
| Reviewed By | Heat Pipe Specialist, Materials Engineer & QA Lead |
| Approved By | Chief Engineer |
| Distribution | Restricted — Engineering, QA, Manufacturing & Field Operations |
| Next Review Date | 2026-10-11 (90 days from issue) |