# Lightweight FPC Harness with Integrated Shielding Film for Analog Signal EMI Protection

Date: 2026-07-20
Reference: FPC light weight wire harness for automobile
TRIZ Principle: Composite materials (Principle #40)
Analysis Method: Functional modeling

# Core Concept

Cross-domain application: Replace traditional shielded copper cables for analog signal routing with a flexible FPC harness wrapped in thin conductive shielding film, grounded to the PCB ground plane via connector contacts.

# 1. Problem — Traditional Shielded Cables

| EMI Source | Frequency Range | Coupling Mechanism |
| --- | --- | --- |
| 48V Buck Converter | 20–100 kHz | Conducted EMI coupling to analog signals |
| Motor ESCs | 8–50 kHz | Conducted EMI coupling to analog signals |

Issues with traditional shielded copper cables:

Heavier (mass increase)

Rigid and hard to route in compact pod topology

Complex routing and bulkier installation

EMI risk to µV-level analog signals (MCT pre-amp, Geophone ADC, QCL Drive)

# 2. Solution — Shielded FPC Harness

Core Modification: Shielding Film + Grounded Contacts

Replace heavy shielded copper cables with:

FPC (Flexible Printed Circuit) main wire — routes µV-level analog signals

Thin conductive shielding film — wraps FPC for EMI attenuation

Grounded connector contacts — bond shielding film to PCB ground plane at multiple points

# Analog Signal Routing (Protected Circuits)

MCT Pre-amp

Geophone ADC (32-bit)

QCL Drive

All three circuits handle µV-level analog signals requiring high integrity and low noise.

# Connector Grounding (Magnified Detail)

| Component | Function |
| --- | --- |
| FPC Conductors | Carry µV-level analog signals |
| Shielding Film (Thin) | Attenuates conducted EMI from power sources |
| Connector Shell (Metal) | Provides mechanical shielding continuity |
| Ground Contact (Multiple Points) | Bonds shield to PCB ground plane for low-impedance return |
| PCB Ground Plane | Reference ground for entire shielding system |

# Key Performance Metrics

| Metric | Value |
| --- | --- |
| Mass reduction | 50–80 g vs. shielded copper cables |
| Wiring harness constraint | Meets ≤200 g total budget |
| Signal integrity | Protects µV-level signals, ensures 32-bit ADC accuracy |
| Routing flexibility | Flexible FPC preserves compact pod layout |
| Modularity | Field-swappable, simple PCB layout |
| EMI attenuation | Shielding film attenuates 8–100 kHz conducted noise |

# Advantages vs. Traditional Shielded Copper Cables

| Traditional (Shielded Cables) | This Solution (FPC Harness) | Benefit |
| --- | --- | --- |
| Heavy copper braid shield | Thin conductive film on lightweight FPC | 50–80 g mass reduction |
| Rigid, hard to route | Flexible routing in tight pod topology | Simplified mechanical layout |
| Bulky connectors | Compact grounded connector contacts | Preserves field-swap modularity |
| Fixed cable lengths | Custom FPC layout per PCB design | Simple PCB layout, no extra enclosures |
| Potential ground loops | Multiple grounded contact points to PCB GND | Controlled low-impedance return path |

# Validation / Parameter Window (To Be Validated)

| Validation Area | Description |
| --- | --- |
| FPC Routing Impedance | Layout & coupling validation to ensure controlled impedance for µV-level analog signals |
| Grounding Topology | Contact resistance measurement and multi-point ground integrity verification |
| Flex Durability | Vibration and bending life testing under operational mechanical loads |
| Environmental Reliability | Tropical humidity exposure and long-term adhesion of shielding film |

# TRIZ Principle Applied

#40 Composite Materials — Transition from homogeneous to heterogeneous materials. The solution combines a flexible polymer substrate (FPC) with a thin conductive shielding film to create a composite harness that achieves functions no single material can provide alone: lightweight flexibility plus electromagnetic shielding. This replaces a homogeneous heavy copper cable with a heterogeneous composite that optimizes mass, flexibility, and EMI protection simultaneously.

# Legend

Solid blue arrow: Signal Path (µV-level Analog)

Dashed blue arrow: Shielding / EMI Attenuation Path

Dashed red arrow: EMI Coupling / Interference Path

Note: All parameters require validation. FPC routing design, connector grounding topology, and flex durability under vibration and tropical humidity are to be confirmed through testing.