# Passive-Active Hybrid Vibration Suppression System

# Saturday, July 11, 2026

# Passive-Active Hybrid Vibration Suppression with Frequency-Separated Isolation and Active Displacement Correction

## Overview

This document describes a cross-domain vibration suppression architecture combining passive elastomeric isolation with active low-frequency displacement correction. The system is designed for airborne optical payloads requiring high-fidelity beam alignment and stable focal plane positioning without the mass, complexity, or integration burden of a full gyrostabilized platform.

## 1. System Concept

### 1.1 Frequency-Separated Vibration Management

- Passive Isolation (50-400 Hz): Existing Copterlab elastomer damper mounts are retained to attenuate high-frequency engine and airframe vibration.

- Active Correction (Low-Frequency 5-50 μm Displacement): Voice-coil or piezoelectric actuators mounted at optical bench nodes counteract residual micro-displacement.

- Condition-Based Separation: High-frequency vibration is mechanically decoupled from low-frequency displacement correction, eliminating the traditional trade-off between isolation effectiveness and optical alignment stiffness.

### 1.2 Cross-Domain Integration

- Optical, mechanical, and control subsystems operate in a coordinated loop.

- Real-time sensor feedback ensures alignment stability across QCL, Sony ILX-LR1, and FLIR Hadron modules.

## 2. Sensors and Feedback Architecture

### 2.1 Accelerometer Placement

- QCL Mounting Node: Tracks micro-radian beam deviation.

- Sony ILX-LR1 Node: Monitors focal plane stability.

- FLIR Hadron Node: Ensures thermal camera alignment.

### 2.2 Control Loop

- Accelerometer data feeds into a vibration controller.

- Controller computes counteracting forces for actuators.

- Actuators maintain ±5 μrad QCL beam alignment and stable camera focal planes.

## 3. Actuator Subsystem

### 3.1 Actuator Types

- Voice Coil Actuators: High bandwidth, smooth force output.

- Piezoelectric Actuators: High precision, compact form factor.

### 3.2 Mechanical Integration

- Modular actuator mounting interface supports <5 min optical bench swap.

- Actuators add 1.0-2.0 kg total mass.

- Fits within ≤12 kg pod budget.

### 3.3 Electrical Integration

- Powered by existing 48V buck converter.

- EMI filtering ensures compatibility with sensitive optical electronics.

## 4. Passive Isolation Layer

### 4.1 Elastomer Damper Characteristics

- Designed for 50-400 Hz vibration attenuation.

- Provides baseline isolation without compromising structural stiffness.

### 4.2 Mechanical Path Decoupling

- Passive dampers handle high-frequency vibration.

- Active actuators correct low-frequency displacement.

- Eliminates shared mechanical path that typically forces design compromises.

## 5. Performance Benefits

### 5.1 Optical Stability

- Maintains ±5 μrad beam alignment for QCL.

- Preserves camera focal plane stability across imaging modalities.

### 5.2 Mechanical Efficiency

- No gyrostabilization platform required.

- Reduced mass, complexity, and maintenance.

### 5.3 Operational Flexibility

- Modular bench interface supports rapid payload swaps.

- Compatible with multi-sensor airborne pods.

## 6. Reference System

Satellite vibration suppression system and method, storage medium and computer program product

- Provides conceptual precedent for multi-layer vibration suppression.

## 7. TRIZ Analysis

### 7.1 Physical Contradictions Addressed

- Need for high isolation vs. need for high stiffness.

- Need for low mass vs. need for high precision alignment.

### 7.2 TRIZ Principles Applied

- Principle #1: Segmentation - Separating vibration management into passive and active layers.

- Principle #40: Composite Materials - Combining elastomeric isolation with precision actuators.

## 8. Summary

This hybrid vibration suppression architecture provides a robust, modular, and lightweight solution for stabilizing airborne optical payloads. By separating high-frequency isolation from low-frequency displacement correction, the system achieves superior alignment stability without the complexity of gyrostabilized platforms.