# Quasi-Zero Stiffness Isolation Layer Between Damper Mounts and Optical Bench

For Decoupled Vibration Attenuation — Two-Stage Isolation Architecture

Sunday, July 19, 2026

# Executive Summary

This document describes the insertion of a quasi-zero stiffness (QZS) isolation device—comprising a bistable beam with spiral springs—between existing Copterlab elastomer damper mounts and the optical bench structure. This creates a two-stage isolation architecture that decouples vibration isolation from optical alignment stiffness, eliminating the shared mechanical path that currently forces a trade-off between isolation effectiveness and alignment stability.

# 1. Problem Statement

The current single-stage mounting architecture creates a shared mechanical path between the engine/airframe vibration source and the optical bench. This forces a fundamental trade-off:

Soft isolation mounts provide good vibration attenuation but allow excessive bench motion, degrading optical alignment.

Stiff mounts preserve alignment but transmit excessive engine vibration to sensitive optical payloads.

Affected optical systems: Quantum Cascade Laser (QCL) requiring ±5 µrad pointing stability, Sony ILX-LR1 focal plane alignment, and FLIR Hadron boresight preservation.

# 2. Solution Principle

Insert a quasi-zero stiffness isolation layer between the existing elastomer dampers and the optical bench. This creates a two-stage vibration isolation architecture:

| Stage | Component | Function |
| --- | --- | --- |
| Stage 0 | Engine / Airframe | Vibration source (broadband, 50–400 Hz) |
| Stage 1 | Copterlab Elastomer Damper Mounts | Primary isolation; attenuates high-frequency engine vibration |
| Stage 2 | QZS Isolation Device (Bistable Beam + Spiral Springs) | Secondary isolation; constant 10³–10⁴ N/m compliance across 50–400 Hz |

# 3. System Architecture

## 3.1 Before: Single-Stage Problem

Engine / Airframe → Damper Mount → Optical Bench (shared mechanical path)

The optical bench is mounted directly to the damper mounts. Vibration transmission and alignment stability are coupled through the same structural path, making it impossible to optimize both simultaneously.

## 3.2 After: Two-Stage Isolation Architecture

Engine / Airframe → Damper Mount → QZS Isolation Device → Optical Bench

The QZS device is inserted as a new isolation layer between the damper mount and the optical bench. The modified interface creates a modular mounting point that allows independent optimization of each stage.

Vibration Transmission Path (Action Path):

| Step | Vibration Path |
| --- | --- |
| 1 | Engine vibration source |
| 2 | Elastomer dampers (Copterlab) — primary attenuation |
| 3 | Bistable isolation device (QZS layer) — secondary attenuation |
| 4 | Optical bench (rigid, >10⁶ N/m internal stiffness) — preserves alignment |
| 5 | Airframe (return path) |

# 4. Key Design Parameters

| Parameter | Value / Target | Notes |
| --- | --- | --- |
| QZS Device Stiffness | 10³ – 10⁴ N/m | Constant compliance across operating band |
| Operating Frequency Range | 50 – 400 Hz | Engine vibration dominant band |
| Optical Bench Internal Stiffness | > 10⁶ N/m | Maintains rigid-body alignment |
| Vibration at Bench Nodes | < 5 µm | Performance target (50–400 Hz) |
| QCL Pointing Stability | ±5 µrad | Preserved by rigid bench structure |
| Added Mass | 0.3 – 0.8 kg | Isolation device only |

# 5. Optical Payload Requirements

| Payload | Critical Requirement | Preservation Mechanism |
| --- | --- | --- |
| QCL (Quantum Cascade Laser) | ±5 µrad pointing stability | Rigid optical bench (>10⁶ N/m) prevents angular deformation |
| Sony ILX-LR1 | Focal plane stability | Bench rigidity maintains sensor-to-lens registration |
| FLIR Hadron | Boresight preservation | Isolation layer prevents bench drift while damping vibration |

# 6. TRIZ / Systematic Innovation Principles

The physical contradiction addressed: the mounting interface must be simultaneously soft (for vibration isolation) and stiff (for optical alignment).

## 6.1 Principle #1 — Segmentation

The single shared mechanical path is segmented into two distinct structural stages:

Stage 1 (soft): QZS isolation device between damper and bench — provides vibration attenuation.

Stage 2 (rigid): Optical bench internal structure — provides alignment stability.

## 6.2 Principle #15 — Dynamics

The system transitions from a static compromise (single stiffness value) to a dynamic, multi-stage architecture where each stage is optimized for its specific function. The bistable beam provides adaptive compliance that remains quasi-zero across the vibration spectrum.

## 6.3 System-Level Separation

By separating the functions in space (different physical layers), the contradiction is resolved without compromise. Vibration isolation and alignment stiffness are no longer competing for the same structural parameter.

# 7. Mass Budget & Operational Constraints

| Constraint | Requirement | Status |
| --- | --- | --- |
| Added Mass (Isolation Device) | 0.3 – 0.8 kg | Within budget |
| Total Pod Mass Budget | ≤ 12 kg | Accommodates added mass |
| Optical Bench Swap Time | < 5 minutes | Modular interface at isolation layer enables rapid swap |

# 8. Validation & Verification Plan

The following items require validation to confirm system performance:

| Validation Item | Method | Pass Criteria |
| --- | --- | --- |
| Isolation Device Dynamics | Shaker table testing + laser vibrometry | Compliance window 10³–10⁴ N/m across 50–400 Hz |
| Long-Term Durability | Accelerated life testing (thermal + vibration cycles) | No stiffness degradation >10% over mission life |
| Thermal / Environmental Effects | Environmental chamber testing (-40°C to +60°C) | QZS characteristics stable across temperature range |
| Optical Performance After Integration | QCL beam pointing + ILX-LR1 MTF + Hadron boresight | All optical specs maintained under vibration |

# 9. Bottleneck Solved

The shared mechanical path that previously caused vibration transmission and alignment instability has been eliminated. The two-stage architecture resolves the fundamental contradiction by:

Providing soft, frequency-tuned isolation at the damper-to-bench interface (10³–10⁴ N/m).

Maintaining rigid internal structure within the optical bench (>10⁶ N/m) to preserve sub-microradian alignment.

Enabling independent optimization of each stage without coupling or trade-off.

# 10. References

[1] Self-adaptive vibration reduction support based on vision measurement technology

[2] Altshuller, G. — TRIZ: 40 Principles of Inventive Problem Solving (Principle #1: Segmentation; Principle #15: Dynamics)