# Spatial Segmentation of ESC and Buck Converter for Near-Field EMI Isolation

Date: 2026-07-20
Reference: Anti-electromagnetic interference design method for wheel-foot humanoid robot in industrial scene
TRIZ Technical Contradictions: Segmentation (#1), Intermediary (#24)

# Key Design Points

| Modification | Detail | Benefit |
| --- | --- | --- |
| ESC relocated | Aft end of pod | Maximizes distance from sensor zone |
| Buck Converter relocated | Forward end of pod | Isolates primary switching noise source |
| Separation increased | ~100 mm → ~250 mm | ~6 dB reduction via 1/r² near-field scaling |
| Carbon fiber partition | Between power & sensor zones | Additional ~4 dB from existing EMI shield layers |
| Combined gain | Spatial + partition | ~10 dB improvement |
| Total attenuation | + passive LC filters | >40 dB cumulative |

# Validation Checklist (To Be Confirmed)

Harness extension adds ~30 g copper (within 200 g budget)

Cable routing must fit pod topology

No obstruction to thermal venting

Avoid new radiated EMI loops from extended harness

# TRIZ Principles Applied

#1 Segmentation — Dividing the pod into independent power and sensor zones

#24 Intermediary — Inserting the carbon fiber partition as a shielding mediator between conflicting fields